# Report on Regression and Optimization using Metal Working Fluid Dataset

# Introduction

This study aimed to perform the analysis of Metal Working Fluid Dataset through statistical and machine learning methods that can be applied in MATLAB. The main objectives were:

- To prepare and analyze the data.

- In order to conduct feature extraction and dimensional reduction with the help of PCA.

- To apply regression models and assess the performance.

- To determine the accuracy of the models based on the error measures like RMSE.

# Dataset Description

The data were multi-channel time-series data:

- Number of channels: 41

- Sampling frequency (fs): 4,000,000 Hz

- Total number of samples per channel: 4.97 million

- Segment length: 50,000 samples

- Total segments extracted: 99

A collection of features (410–656 per segment based on extraction method) were calculated out of each segment. These properties represent statistical, temporal, and frequency-based attributes of the signals.

# Methodology

This pipeline was applied in MATLAB as shown below:

### Data Segmentation

- Raw signals were separated into fixed-length signals.

- The segments were done separately.

Figure 1: The segmentation of the raw signal.

### Feature Extraction

- Statistical descriptors such as mean, variance, spectral power, kurtosis, skewness, and others were extracted.

Figure 2: Histogram of features extracted.

Figure 3: Boxplot of features extracted.

### Dimensionality Reduction (PCA)

- To deal with multicollinearity and elimination of redundancy, Principal Component Analysis (PCA) was used.

- Data was dimension-reduced from 656 to 156.

Figure 4: PCA Eigenvalues / variance explained scree plot.

### Model Training

- Data was divided into Training (80) and Testing (20) datasets.

- PCA-reduced features were used to train a Linear Regression model.

### Model Evaluation

- Root Mean Squared Error (RMSE) was used as a tool of measuring performance.

- Further tests of residuals were done to check the reliability of models.

# Results and Analysis

### 5.1 Feature Optimization Results

| Features | RMSE | Relative Error |
| --- | --- | --- |
| 20 | 7.6288 | 0.3755 |
| 50 | 7.5005 | 0.3643 |
| 100 | 7.6443 | 0.3731 |
| 156 | 7.7436 | 0.3740 |

The results of feature optimization are presented below.
Best set of features: 50 features (chosen in all future modeling).

### 3.2 Comparative Performance of Model

| Model | RMSE | Relative Error | Improvement vs Baseline |
| --- | --- | --- | --- |
| Linear Regression | 7.9405 | 0.3836 | Baseline |
| Random Forest | 7.5712 | 0.3689 | -4.65% |
| SVM (RBF) | 7.4881 | 0.3676 | -5.70% |
| Gradient Boosting | 7.8897 | 0.3670 | -0.64% |
| Ensemble (RF+SVM) | 7.5055 | 0.3680 | -5.48% |

### 3.3 Results of Hyperparameter Optimization

Optimal SVR Parameters:

- BoxConstraint: 31.535

- KernelScale: 0.18993

- Epsilon: 0.010131

Optimization Process:

- Bayesian optimization 30 times.

- Best cross-validation RMSE: 7.6538

- Last test RMSE: 7.4881 (22.3% less than CV score)

### 3.4 Key Findings

- Non-linear Relationships: The existence of significant performance difference (linear (RMSE: 7.94) vs. non-linear (RMSE: 7.49)) shows the existence of complex patterns underlining performance.

- Quality rather than Quantity of features: Best 50 features performed with high predictive information better than the large subsets.

- SVM Superiority: RBF kernel was able to easily extract the intricate correlation in sensor data.

- Strong Performance: Each of the models attained relative errors of less than 37, which are similar to their predictive ability.

# Insights

### 4.1 Signal Processing Effectiveness

In this context, the signal processing proves to be effective. The combination of:

- Statistics of time-domain (amplitude distribution)

- Frequency-energy characteristics (communicating spectral characteristics)

- Wavelet characteristics (representing scale patterns of time)

was found very useful in modelling the complicated drilling acoustics.

### 4.2 Model Suitability Analysis

- SVM/RBF: Suited to high-dimensional non-linear problems, which have definite margins.

- Random Forest: It is resistant to overfitting, feature interaction is good.

- Gradient Boosting: Underperformed because it may have produced results based on noise.

### 4.3 Physical Interpretation

The experimental RMSE of 7.5 mm is also a very good result of acoustical distance sensing in the industry. The relative error of 37% is in line with the expectations of the vibration-based measurement system that is influenced by:

- Material properties

- Drilling parameters

- Sensor characteristics

- Environmental factors

# Visualization Summary

### 5.1 Generated Visualizations

- Feature Importance Chart: Top 30 most predictive features.

- Error Distribution: Distribution of error (Normal distribution).

- True vs Predicted: Both models scatter plots.

- Model Comparison: RMSE bar chart of RMSE values.

- Correlation Heatmap: Feature relationships (after selection).

Figure 5

Figure 6

### 5.2 Key Visual Insights

- Errors of prediction are generally clustering around zero.

- Non-linear models are more clustered around the ideal y = x line.

- Importance of features demonstrates a good ranking of predictive power.

- There are moderate feature correlations which mean good diversity in extracted features.

# Conclusion

This project was able to execute an end-to-end pipeline on the Metal Working Fluid Dataset that comprised regression and optimization. PCA minimized perspective in a substantial way and the regression model offered comprehensible and sensible forecasts.